Brillion

TO USERS OF THE NEW CASTLE CONSERVATION DISTRICT SEEDER-SS8-8’ Wide

Needs a 55 hp tractor with hydraulics to raise and lower traveling wheels.  Planter can be pulled behind a pickup truck.  Supply your own hitchpin.  Calibration of the drill according to the "Operators Manual" found in the toolbox gives the best available estimate for the specific seeding rates.   The New Castle Conservation District accepts no responsibility for the accuracy of the setting.

Transport:

- Raise machine fully and pin transport lock in place on each lift cylinder.

Before using the drill:

	Disengage each transport lock and replace pin on each lift cylinder.

- Check for any loose or broken parts.  Check Tires.

	Install the safety chain.

	Set the acreage meter to "O".

	Operate at a ground speed of 4-6 mph.

- Grease zerks daily (see page 12)

After using the drill:

	Lock up the openers.

	Clean out seed from all hoppers and tubes.  Vacuum or blow out the hoppers, clean out feed cups by dropping latch all the way down, and shake out the tubes.  If you used the small seed hopper, remove the drive chain and place in tool box.  There is a $50 hourly charge for not cleaning seed out of the drill.

	Read the acreage meter.

	Call Elizabeth Hook at 832-3100 extension 8970 to report completion and acres.  The next farmer will pick it up from you. The District will send you a bill.

Repairs Needed:

	Call Elizabeth Hook at 832-3100 extension 8970.

	If she is not available and parts or repairs are needed call Atlantic Tractor at 410-275-2195 in Cecilton, MD.

Cannot back up with this equipment